GLOBAL NUMERICAL ANALYSIS OF A COUPLED 2D NAVIER–STOKES AND ADVECTION–DIFFUSION SYSTEM

Turakulov J.A

jahondasturchi@gmail.com

Annotation. This article explores a bounded system consisting of an incompressible two-dimensional Navier-Stokes equation and an advection-diffusion transport equation. When determining the velocity and pressure fields of a liquid using the Navier-Stokes equations, the distribution of concentration in the liquid under the influence of the flow is observed using the advection-diffusion equation. A bounded system is solved using an explicit scheme based on the finite difference method. The proposed discretization method provides stability and accuracy in time and space. Numerical experiments show that the model can effectively describe convective-diffusion processes in two-dimensional flows.

Key words: Navier-Stokes equations, advection-diffusion equation, Pressure gradient, numerical solution.

In recent years, precise modeling of fluid flows, production fluid filters, and associated transport processes has become increasingly important in many modern problems of ecology, energy, biotechnology, the pharmaceutical industry, and industrial engineering. Especially in the field of computational fluid mechanics, the need for coupled flow-transport models is growing. The incompressible Navier-Stokes equations are the main mathematical model describing the flow dynamics in fluid mechanics. In many physical and engineering problems, the movement of matter, heat, or pollutant particles along with the flow is modeled by the advection-diffusion equation. Therefore, the study of bound systems consisting of the Navier-Stokes equation and the advection-diffusion equation is of great scientific and practical importance. Such systems represent a complex interaction of convective-diffusion transport processes. In this work, this dependent model is considered in a two-dimensional case. The system is solved numerically based on the finite difference method. The stability and accuracy of the discretization circuit are ensured by parameters selected in time and space. The proposed approach allows us to clearly express the relationship between flow and scalar transport. The obtained conclusions serve as a basis for a more in-depth analysis of transport processes in fluid flows.

[1] This article is devoted to the study of three-dimensional compressible Navier-Stokes equations in unstructured networks. The approach used in the scientific work is based on the separation of the convection and diffusion parts. Convective flow terms are calculated using Godunov's method. A new finite-volume scheme for calculating the diffusion part is presented. Numerical results are presented to verify the effectiveness of the newly developed method.

[2] In this work, the Cauchy problem, consisting of an isentropically compressible Navier-Stokes system, associated with the non-pressure three-dimensional Navier-Stokes equations, was investigated. The system is interconnected by a member of the resistance force, and the initial data are considered as small perturbations of the constant equilibrium state. The global existence of strong solutions has been proven. Due to the non-pressure structure, the density was analyzed only by means of the transport equation, and estimates of decomposition over a specific time for velocity were obtained. Lyapunov density stability in Sobolev space was determined using spectral analysis, low-high frequency resolution, and time-measured energy.

[3] In this article, the 3D Navier-Stokes equations, along with the slip-friction boundary conditions, are studied using precise spectral analysis. In the study, a simple geometric domain between two parallel planes is considered, with conditions varying over time along the two planes. In the context of boundary layer theory, this condition requires special processing of the functional method and makes it possible to accurately describe the eigenvalues and eigenfunctions of the Stokes problem. These eigenvalues are used to determine infinite-dimensional data classes leading to a solution in the general case for evolutionary Navier-Stokes equations. Thus, the spectral approach expands the mathematical analysis of boundary-layer flows.

[4] In this article, the uncompressible Navier-Stokes/Can-Hilliard system under the generalized Navier boundary condition, describing the interaction between binary material and the walls of a physical domain, is investigated. The problem is posed in the channel with dynamic boundary conditions. The existence of strong solutions over a total time for the initial-boundary value problem in the domain of a two-dimensional channel has been proven. Also, the uniqueness of these solutions has been established. The obtained results serve as a mathematical justification of fluid-phase interactions.

[5] In this article, the Kahn-Hilliard equation is widely used in modeling two-phase flows due to its implicit representation of the interface and favorable expansion to higher dimensions, but it has drawbacks such as the loss of mass of small droplets. In this work, the Kahn-Hilliard equation with non-uniform motion is considered in conjunction with the Navier-Stokes equations. The system is effectively solved on the basis of the finite difference method, and the chemical potential is discretized by a modified Laplacian central difference scheme. The proposed approach provides high accuracy, allowing the application of longer time stages. Through numerical experiments and experimental comparisons, the new model has shown that it retains mass better and small droplets are more stable.

[6] In this article [15] J. Turakulov solved numerically the change in the physical properties of a liquid when flowing through a single-layer medium by calculating the processes of colmatation and suffusion. Analysis of the literature showed that the filtration process of multicomponent and low-concentration solutions was studied in detail, taking into account the simultaneous process of colmatation and suffusion, the operating modes of the filter, changes in the filtration rate of the liquid, and the multilayer nature of the filter column.

Analysis of the scientific works studied above shows that solving the transport problem in the flow of liquid with high accuracy remains one of the urgent problems in everyday life and production.

Problem statement

The following mathematical model consists of the Navier-Stokes equation and the advection-diffusion equation. Fluid flow and the change in its concentration allow us to observe it at any time and in any space.

(1)

Initial and boundary conditions:

(2)

(3)

(4)

Here  - concentration,  - flow velocities,  - diffusion,  - density,  - dynamic viscosity,  - pressure.

Numerical solution.

It is impossible to solve the above problem analytically so to solve it numerically, we introduce the following discrete domain.

(5)

We discretise the above mathematical model in the explicit scheme of the finite difference method.

(6)

(7)

Predictor step in solving the Navier-Stokes equation:

(8)

(9)

Let's find the pressure using the Gauss-Zeidel method:

(10)

The next step of the flow velocity in time is found through the corrector step.:

(11)

(12)

A computational algorithm has been developed for conducting a computational experiment for the numerical solution of the problem, synthesis of the main parameters, their ranges of change, and their prediction.

Results.

In order to study the fluid flow and the distribution of its concentration, numerical calculations were carried out in the computational cluster, the results of which are presented in Fig. 2-6.

Computational experiments were conducted with the following initial data:

[1] Abdelwahed, M., Bade, R., Chaker, H., On the study of three-dimensional compressible Navier–Stokes equations. Bound Value Probl 2024, 84 (2024). https://doi.org/10.1186/s13661-024-01893-9

[2] Zhang, Y., Yu, M. & Tang, H. Global strong solution of the pressureless Navier-Stokes/Navier-Stokes system. Acta Math Sci 45, 1045–1062 (2025). https://doi.org/10.1007/s10473-025-0316-2.

[3] Berselli, L.C., Falocchi, A. & Sannipoli, R. On a 3D Stokes eigenvalue problem under Navier slip-with-friction boundary conditions and applications to Navier–Stokes equations. Z. Angew. Math. Phys. 76, 175 (2025). https://doi.org/10.1007/s00033-025-02556-0

[4] Ding, S., Li, Y. & Yan, Y. Global strong solutions to Navier-Stokes/Cahn-Hilliard equations with generalized Navier boundary condition and dynamic boundary condition. Acta Math Sci 45, 2305–2329 (2025). https://doi.org/10.1007/s10473-025-0601-0

[5] Shen, M., Li, B.Q. & Yang, H. A Cahn–Hilliard–Navier–Stokes system with degenerate mobilities: a finite difference solution. Engineering with Computers 42, 38 (2026). https://doi.org/10.1007/s00366-025-02257-2

[6] Равшанов Н., Туракулов Ж. “Математическое моделирование процесса фильтрования малоконцентрированных растворов через пористую среду” // Информационные технологии моделирования и управления – 2024. – №3(137).